Worked Example: Waste and Emissions -- Coastal Country

SEEA-CF framework: tracking flows of residuals from the economy to the marine environment  |  Year: 2022

Illustrative mid-income coastal country (~15 million population, mixed agricultural/industrial/service economy). Data compiled from environmental ministry reports, port authority records, and UNEP Global Wastewater Initiative.

Table 9: Water Emissions to the Marine Environment (tonnes/year)

| Source sector | Nitrogen (tonnes N/yr) | Phosphorus (tonnes P/yr) | BOD (tonnes O₂/yr) | Heavy metals (tonnes/yr) | Total load (tonnes/yr) |
| --- | --- | --- | --- | --- | --- |
| Agriculture (fertiliser runoff, livestock) | 12,400 | 3,100 | 8,200 | 15 | 23,715 |
| Manufacturing (food processing, textiles) | 1,800 | 420 | 6,500 | 85 | 8,805 |
| Sewerage: treated (secondary treatment) | 2,200 | 380 | 1,800 | 8 | 4,388 |
| Sewerage: untreated (direct discharge) | 8,600 | 2,100 | 18,500 | 12 | 29,212 |
| Households (direct coastal discharge) | 1,400 | 350 | 3,200 | 2 | 4,952 |
| Aquaculture (fish/shrimp farms) | 2,100 | 680 | 4,800 | 5 | 7,585 |
| Other (stormwater, recreational vessels) | 800 | 120 | 1,200 | 3 | 2,123 |
| TOTAL | 29,300 | 7,150 | 44,200 | 130 | 80,780 |

Interpretation:

The largest pollution source by volume is untreated sewage discharge (29,212 tonnes/yr, 36% of total load), highlighting the critical infrastructure gap. Agriculture is the largest source of nitrogen (12,400 tonnes N/yr, 42% of total N) due to fertiliser runoff. Manufacturing contributes the most heavy metals (85 tonnes/yr, 65%). BOD load is dominated by untreated sewage (18,500 tonnes, 42%), which drives coastal deoxygenation.

Table 10: Solid Waste Entering the Marine Environment (tonnes/year)

| Waste type | Land-based sources | Sea-based sources | Total (tonnes/yr) | % of total |
| --- | --- | --- | --- | --- |
| Plastics: macroplastic (packaging, bags, bottles) | 8,100 | 1,300 | 9,400 | 52% |
| Plastics: microplastic (fibres, pellets, fragments) | 1,200 | 600 | 1,800 | 10% |
| Fishing gear (nets, lines, traps) | 0 | 2,000 | 2,000 | 11% |
| Ship waste (bilge, cargo residue) | 0 | 850 | 850 | 5% |
| Sewage sludge | 1,500 | 0 | 1,500 | 8% |
| Organic waste (food, vegetation) | 1,800 | 200 | 2,000 | 11% |
| Other (glass, metal, construction debris) | 350 | 100 | 450 | 3% |
| TOTAL | 12,950 | 5,050 | 18,000 | 100% |

Interpretation:

The most persistent pollutant is plastic waste, comprising 62% of total marine solid waste (macro + micro combined: 11,200 tonnes). Land-based sources dominate (72%), but sea-based sources are significant (28%), mainly from fishing gear (2,000 tonnes, 11% of total) and commercial shipping. Fishing gear, though smaller in volume than packaging plastics, causes disproportionate ecological harm through ghost fishing. The 52% macroplastic share aligns with global estimates (Jambeck et al. 2015).

Table 11: Air Emissions from Ocean Industries (tonnes/year)

| Ocean sector | Fuel consumed (tonnes/yr) | CO₂ (tonnes/yr) | SO₂ (tonnes/yr) | NOx (tonnes/yr) | PM₂.₅ (tonnes/yr) |
| --- | --- | --- | --- | --- | --- |
| International shipping (IMO-registered) | 185,000 | 573,500 | 9,250 | 14,800 | 1,295 |
| Domestic shipping and ferries | 42,000 | 130,200 | 2,100 | 3,360 | 294 |
| Fishing fleet (motorised vessels) | 28,000 | 86,800 | 1,400 | 2,240 | 196 |
| Offshore oil and gas (platforms, supply) | 15,500 | 48,050 | 775 | 1,240 | 109 |
| Port operations (cranes, vehicles, tugs) | 8,500 | 26,350 | 425 | 680 | 60 |
| TOTAL | 279,000 | 864,900 | 13,950 | 22,320 | 1,954 |

Interpretation:

Maritime shipping (international + domestic) dominates air emissions from the ocean economy, accounting for 81% of CO₂, 81% of SO₂, and 81% of NOx. International shipping alone contributes 66% of total ocean-sector CO₂. The fishing fleet, though smaller in absolute emissions, operates closer to shore and in ecologically sensitive areas. SO₂ emissions are a particular concern for port-adjacent communities and are subject to IMO 2020 sulphur regulations (0.50% m/m fuel sulphur limit).

Policy linkage:

These tables directly support SDG 14 (Life Below Water) indicator 14.1.1 (coastal eutrophication / floating plastic debris density) and can feed into national GHG inventory reporting under UNFCCC. The separation of treated vs. untreated sewage helps prioritise wastewater infrastructure investment.